# 2.4 Computational Logic

### FAQ

| Question |  |  |
| --- | --- | --- |
| 1 | How many inputs are there in a truth table? | 2number of inputs |
| 2 | What is the difference between XOR and OR? | XOR is Exclusive OR – and not part of the GCSE Syllabus. OR ouput is TRUE whenever one on more of the inputs is true/on. XOR is only TRUE if only one input is on exclusively. If BOTH inputs are on for an XOR, the resultant output is FALSE. |
| 3 | Do combinations of Logic Gates follow Mathematical Laws for brackets? | Yes, combinations of gates within brackets are calculated first. |
| 4 | What level of complexity will learners be expected to calculate? | Learners should be able to calculate 2nd Level problems; i.e. output(s) of a gate(s) form the input of another gate (s) |
| 5 | Does it matter if I use 1/0 or T/F? | No. Candidates could reply with either, but see Appendix 5e for exam board notation and follow this guide for understanding how questions will be presented in an exam paper. |

### Common Misconceptions

| Section | Error | Explanation |
| --- | --- | --- |
| Truth Tables: Inputs | Not including all inputs in complex circuits. | Students often jump straight into calculating the result of a gate in isolation and then using that to calculate the results of the next gate, without considering that it actually reduces the number of inputs in the circuit. e.g. A AND (B OR C) |
|  |  | Students are likely to create a B OR C truth table – giving 2 outputs i.e. Gate result is either 1 or 0. B; C; B C. 0; 0; 0. 0; 1; 1. 1; 0; 1. 1; 1; 1. This then leads to the final table being erroneous. A; B C; A AND (B C). 0; 0; 0. 0; 1; 0. 1; 0; 0. 1; 1; 1. Solution: Ensure that students remember the 2n rule for number of rows in truth table as demonstrated within the presentation. |

OCR Resources: the small print

OCR’s resources are provided to support the teaching of OCR specifications, but in no way constitute an endorsed teaching method that is required by the Board, and the decision to use them lies with the individual teacher.   Whilst every effort is made to ensure the accuracy of the content, OCR cannot be held responsible for any errors or omissions within these resources.

© OCR 2016 - This resource may be freely copied and distributed, as long as the OCR logo and this message remain intact and OCR is acknowledged as the originator of this work.

OCR acknowledges the use of the following content: n/a

Please get in touch if you want to discuss the accessibility of resources we offer to support delivery of our qualifications: resources.feedback@ocr.org.uk

We’d like to know your view on the resources we produce.  By clicking on ‘Like’ or ‘Dislike’ you can help us to ensure that our resources work for you.  When the email template pops up please add additional comments if you wish and then just click ‘Send’.  Thank you.

If you do not currently offer this OCR qualification but would like to do so, please complete the Expression of Interest Form which can be found here: www.ocr.org.uk/expression-of-interest